Last Revision: 16DE2005

1969 THRU 1979 C3 CORVETTE STANDARD (NON-ADJUSTABLE) STEERING COLUMN DISASSEMBLY & REPAIR INSTRUCTIONS

PAPER #2

Disassembly and Repair Instructions Addressed in this Paper

Degree of Difficulty   Page

## DROP OR REMOVE STEERING COLUMN FROM VEHICLE	 Moderate	 2 & 6

REPLACE TURN SIGNAL SWITCH				 Moderate      2, 3 & 4

TURN SIGNAL SWITCH WIRE COLOR AND GAGE		 Moderate	    3

REPLACE/ADJUST DIMMER SWITCH				 Moderate	    5

REPLACE/ADJUST IGNITION SWITCH	 			 Moderate	 5 & 6

# How the Paper is Setup

There are a limited number of steering column service procedures that are addressed in this paper.  This paper starts at the point where the steering wheel, horn parts, shaft lock, ignition lock cylinder, and the key warning buzzer have been removed from the steering column.  The turn signal switch has been detached from the column housing and has been moved up and out of the way (the wiring is still in the column.)  All of these operations were described in detail in Paper #1.

There are several reasons why you would want to drop or remove your steering column from your vehicle.  First, in order to work on the ignition switch or the dimmer switch you will find that they are totally inaccessible unless you drop the column.  Second, for other major work on the column, it is far easier to do it on a bench than in the car.

This paper makes reference to various line drawing descriptions.  They are included on several pages entitled C3 Standard Steering Column Page #1 & #2.  The steering column installation parts are called out on C3 Steering Column Installation Diagrams.  Also there is a schematic drawing entitled C3 Standard Steering Column Blowup.  Most steering column and installation parts will be called out with a reference numbers and letters from these drawings.  The drawings are all available from the author or from the host websight.  You will find these pictures and descriptions to be most helpful when working on your steering column.

# Types of C3 Steering Columns Addressed in this Paper

This paper addresses the 1969 through 1979 C3 second generation energy absorbing, function locking, standard (non-adjustable) steering columns.  It does not address the 1968, first generation, energy absorbing, standard steering column.

A word of caution:  DISCONNECT THE BATTERY when you work on the steering column.  With the steering column partially disassembled it is possible to inadvertently move the ignition switch to the “start” position.

# Instructions for Dropping and/or Removing the Steering Column from the Vehicle

If your C3 has a steering column back drive system (all 1969 through 1976 models), you should begin by first opening the hood and looking directly under the brake booster.  You should see the end of the steering column sticking through the front of dash.  There should be a lever on the lower end of the steering column with a cable attached to it.  Remove the cotter pin #5 and washer #6.  Disconnect the back drive cable #2 from the lever.  There is also a metal plate (M) attached to the dash.  The cable sheath is attached to that metal plate (You can leave the cable attached to the plate.)  This plate also holds a welded stud and a carriage bolt (N) that secures the lower end of the steering column.

If you have a manual transmission, you will also find a fairly heavy clutch pedal return spring attached to the plate and to the Z-bar.  You should detach one end of the spring at this time.  Hint!  Detach the clutch pedal push rod from the Z-bar.  This will allow the Z-bar to rotate rearward and take some tension off the spring.

Next, you can either remove the two nuts (G) and lockwashers (H) that connect the flexible coupling to the steering column flange (F) or you can mark the column flange to steering shaft location with chalk or crayon and remove the column flange pinch bolt (S).

Go back inside the car and remove the two nuts (R) that attach the steering column at the floor.  Now go back under the hood and pull the lower column plate (or back drive cable plate) away from the dash.  With a back drive column you should now see a slot in the dash that should align with the lever on the column and allow the column to slide back inside the car.  It is most helpful to have an assistant guide the lower end of the column from the engine side while you perform the following procedures.

Now, go back inside and unfasten the two vertical bolts (K) that hold the column up under the dash.  You should now be able to carefully rotate and lower the steering column.  Make sure on 1976 and earlier columns that you have the back drive lever all the way up and that it is aligned with the slot in the front of dash.   Now pull the column straight back, disconnecting the column from the flexible coupling.  This will allow the column to drop so that it will rest on your front seat.

You can now disconnect the body harness connectors from the column ignition switch.  There are three plastic tabs located on the body wiring harness connectors.  The standard column switches have the tabs on top.  You will need to depress them to get them to disengage.  Disconnect the harness connector from the dimmer switch (if so equipped).  The switches and/or the steering column can now be removed from the car.

Removing the Turn Signal Switch – Description #2

Begin by wrapping the edge of the plastic turn signal switch connector and the wires that lead back into the column with tape to keep them in a line.  This will help prevent the connector from cocking and snagging as you pull the turn signal switch wires up through the column.  You want the tape to form a very smooth “bullet” nose between the connector and the wiring.  However, you do not want the tape to be very thick!

Removing the Turn Signal Switch (Continued)

Helpful Hint!  Tie a piece of light but strong wire or some twine to the connector to be used as a “tracer” wire to help during reassembly.  Place the lower column back drive lever in the full “down” position.  Now, pull the switch straight out, guiding the wiring harness and connector out at the same time.  Leave the “tracer” wire or twine in the column so that you can tie it to the new signal switch connector and help direct the wiring back down through the column.

# Alternate Method of Feeding Switch Wires Through the Column

| Some people have been successful using the following procedure; disengage and remove the individual switch wires and metal contacts from the plastic “harmonica” connector. Straighten a heavy paper clip or use a very thin blade screwdriver. Insert it into the "harmonica" connector from the contact side to disengage each wire and contact. There should be a small molded square channel in the connector that will guide you to a metal tang on the contact that holds it in place. Once you depress the tang and pop the wire and contact out of the connector, you should take a small knife blade and bend the tang back out so that it will engage the connector correctly upon reinstallation. The turn signal switch wiring can now be fed separately through the column. YEARS | WIRE COLOR AND GAGE | CONNECTOR POSITION |
| --- | --- | --- |
| 1969-82 | White 18 gage | P |
| 1969-82 | Dark Green 18 gage | N |
| 1969-82 | Yellow 18 gage | M |
| 1969-82 | Purple 18 gage | L |
| 1969-82 | Brown 18 gage | K |
| 1969-82 | Dark Blue 18 gage | J |
| 1969-82 | Light Blue 18 gage | H |
| 1969-82 | Black 18 gage | G |
| 1969-82 | Light Green 20 gage | F |
| 1969-82 | Tan 20 gage | E |
| 1980, 81, & 82 | Black 18 gage | C |
| 1980, 81, & 82 | Orange 18 gage | B |
| 1980, 81, & 82 | Brown 18 gage | A |

Make sure that you reinstall your switch wires into the harmonica connector in the correct order, so that when you reassemble them, all your lights, horn, and buzzers will work correctly.

If all you wanted to do was replace the turn signal switch, the installation procedure follows directly.

If you are continuing to disassemble the column you will need to remove the ignition and dimmer switches from the column and then proceed to Paper #3.

# Reinstalling the Turn Signal Switch

# Set the back drive lever all the way “down”.  Pull the wiring down through the housing with the aid of the tracer wire.  Be sure it feeds under the mounting bracket.  If the column is out of the car, you might find that feeding the wires and connector is easier if someone holds the column vertical.  You will note that the cavity inside the column for the wires to feed extends from about the 4 to 5 o’clock positions.  Have the connector at 5 o’clock and the wires at 4 o’clock.  Use the tracer wire and push down on the connector with a long, thin screwdriver.

Reinstalling the Turn Signal Switch (Continued)

Please return to Paper #1 (C3 Standard Column D&R Paper #1, page #6) to complete the installation of the turn signal switch into the column.  The reinstallation procedure for the entire steering column into the vehicle follows at the end of this paper.

# Replace/Adjust Ignition and/or Dimmer Switches

In order to gain access to the ignition switch and/or the dimmer switch, the steering column must be dropped down from the dash or removed from the car entirely.  It is best to first remove the steering wheel since the column will not drop very far because the steering wheel rim hits the car seat.  The various types of C3 steering wheels and their removal are discussed quite thoroughly in Paper #1.

# General Information

There are two different ignition switches.  The standard (non-adjustable) steering column ignition switch works in the opposite direction from the T&T or tilt steering column switch.  In other words, the standard column has a rod that pushes down on the standard ignition switch to move it into the “start” position.  The adjustable column pulls up on a rod to move its ignition switch to the “start” position.   Although either switch will connect to your vehicle wiring harness, the switches are unique and not interchangeable.  However, you will need to twist the connector on your vehicle wiring harness 180 degrees to convert from a standard (non-adjustable) steering column to a T&T column.

The headlight dimmer switch was mounted on the floor from 1969 through 1976.  It was placed on the steering column in 1977 and continued through 1982.

On all Corvette steering columns from1969 through 1976 there is a lever on the lower end of the column (out under the hood, just under the brake booster.)  It is connected to a cable that runs to the transmission.  We call this cable the back drive cable.  If your car has an automatic transmission, it activates when you shift into “Park.”  If it is a manual transmission, it activates when you shift to “Reverse.”  The ignition switch and lock cylinder must be in the “Off-Unlock” position and the steering column lower lever in the full “up” position when assembling the ignition switch to the column.

1977 through 1979 Corvette standard columns have a small key release lever next to the ignition lock cylinder that the driver has to trip in order to be able to rotate the ignition key past the “Off-Unlock” to the “Off-Lock” and “Accessory” positions.  This later design does not require a back drive cable.  This key release column requires that you also set the ignition switch and lock cylinder in the “Off-Unlock” positions in order to be set correctly.

# Removing the Dimmer Switch

Disconnect the electrical connector from the dimmer switch.

The lower dimmer switch attachment (closest to the floor) is with a nut.  The nut screws onto a stud.  The stud holds the ignition switch in position.  If you are only going to work on the dimmer switch and not the ignition switch, make sure that you only remove the nut and NOT loosen the stud.  Otherwise you will have to adjust the ignition switch as well.

Now remove the screw on the upper part (steering wheel side) of the dimmer switch.  You can now disengage the dimmer switch from the actuator rod and remove it.

# Installing and Adjusting the Dimmer Switch –

# Std Column Description #5

Insert the dimmer rod into the switch.  Loosely install the screw in the upper attaching hole and snug the nut on the stud.  Depress the switch slightly and insert a 3/32” drill into the switch as shown to remove lash.  Tighten the screw and nut to 35 in-lbs.  Remove the drill and check the dimmer switch function by actuating the turn signal lever.  You should hear a sharp snap.

Removing the Ignition Switch

On cars equipped with column mounted dimmer switches, the dimmer switch will have to be removed first and then the ignition switch.  Follow the above procedures to remove the dimmer switch.

Disconnect the electrical connector and remove the ignition switch from the column by removing two screws and pulling the switch off the column actuator rod.  You may need to twist the switch 90 degrees to get it off the actuator rod.  If further disassembly of the column is necessary, please download paper #3 and continue your repairs.

# Installing and Adjusting the Ignition Switch –

# Std Column Description #5

In order to correctly install the ignition switch it is necessary for the lock cylinder to be installed in the column and placed in the “Off Unlock” position.

On all back drive columns (1969 through 1976), reach down to the lower column lever and move it down to its lowest position.  Now rotate the lock cylinder all the way counterclockwise until it stops.  This will be the “Off Unlock” position.

On key release columns, just rotate the lock cylinder all the way counterclockwise until it stops.  DO NOT actuate the key release lever.  This will place it in the “Off Unlock” position.

You now need to place the ignition switch in "Off Unlock" position.  You do this by moving the slider all the way to the end of travel (“Accessory”) where it should detent and stay in that position.  If in doubt, the other extreme will be the "Start" position and you will feel a spring return.  Now move the slider two detent positions back from the “Accessory” position.

#### Installing and Adjusting the Ignition Switch (Continued)

Fit the actuator rod into the slider hole and assemble it to the column with the two screws.  Lightly push the switch down the column (away from the steering wheel), to take out lash in the actuator rod, and tighten the mounting screws.  Caution should be exercised to prevent moving the switch out of the “Off Unlock” detent.  Tighten the screws to 35 in-lbs.  Reinstall and readjust the dimmer switch if so equipped.

# Reinstalling the Steering Column into the Vehicle

Move the lower column lever back to the up position.  Carefully slide the column back through the dash hole.  Insert the column flange onto the two flexible coupling bolts (the bolts are different sizes so the flange will only go on one way.)  Tighten the nuts and lockwashers to 20 ft-lbs.  Reconnect the electrical connectors to the ignition, turn signal, and the dimmer switches.

Some people have reported that when they replaced their turn signal switch they found that the “harmonica” connector from the new switch would not snap correctly onto the vehicle wiring connector.  The problem can easily be corrected by swapping your original connector onto wires from the new switch.  Instructions were included earlier on page #3 to remove and reinstall the wires.

Snug but do not tighten the column in place with the two screws up under the dash.  Install the lower column plate with its stud and carriage bolt.  Now check the alignment of the flexible coupling pins in the column flange slots.  They should be central.  You may need to move the lower column and outside plate around in order to make the pins central.  Then reattach the floor fasteners and torque to 10 ft-lbs.  Rotate the steering wheel 90 degrees and again visually check if the flexible coupling pins are central in the flange slots.  Loosen the floor fasteners and readjust the column as necessary.  Torque the two vertical column screws to 15 ft-lbs.

Reattach the AC duct, the crossover bar, and lower dash panels.

With the column lever all the way up, reattach the transmission back drive cable (manual transmission in “Reverse”, automatic in “Park”) if so equipped.

Refer to paper #1 for reinstalling the shaft lock, steering wheel and horn parts.

StdC3ColumnD&R#216DE05.doc

JIML82@aol.com