Last Revision 15JA03

1969 THRU 1972 GM A-CAR STANDARD (NON-ADJUSTABLE)  STEERING COLUMN DISASSEMBLY & REPAIR INSTRUCTIONS

### PAPER #3

# Disassembly and Repair Instructions Addressed in this Paper

Degree of Difficulty   Page

# REPLACE RACK AND/OR SECTOR	 			 Moderate	     2

# How the Paper is Setup

There are a limited number of steering column service procedures that are addressed in this paper.  This paper starts at the point where the steering wheel, horn parts, shaft lock, ignition lock cylinder, key warning buzzer, turn signal switch, and ignition switch have been removed from a standard steering column.  The steering column has been removed from the car and is on a work bench.  All of these operations were described in detail in Papers #1 & #2.

This paper makes reference to various line drawing descriptions.  They are included on several pages entitled GM A-Car Standard Steering Column Page #1 & #2.

The steering column installation is shown on a drawing entitled GM A-Car Steering Column Parts and Mounting.

Also there is a schematic drawing entitled GM A-Car Standard Steering Column Blowup.  Most steering column and installation parts will be called out with a reference numbers and letters from these drawings.

The drawings are all available from the author or from the host websight. You will find these pictures and descriptions to be most helpful when working on your steering column.

# Page 4 of this paper includes five pictures labeled FIG. 2, 3, 4, 5, & 6.  They should assist in disassembling and repairing the column head, the sector, rack, and lock bolt.

# Types of Steering Columns Addressed in this Paper

This is a generic paper for all standard steering columns that were installed in General Motors 1969 through 1976 A-car models.

Column Shift Models

Drive out the upper shift lever pivot pin #13 and remove the shift lever #11.

Remove Housing and Shroud from Column Jacket – Description #4

Remove the four hex head screws #35 securing the housing to the column jacket #63.  Pull the assembly from the jacket (See FIG. 2).

On floor shift models, tip the housing and shroud assembly upside down.  Remove three screws and separate the shroud #12 from the housing.  On column shift models the shroud snaps into the gearshift lever housing #14.

Remove the thrust cap #23 from the lower side of the housing (See FIG. 3).

# Replace Rack and/or Sector

Remove the rack and lock bolt assembly #20 & #22.  If required, pull the rack away from the lock bolt to separate.  Remove the rack preload spring #24 (See FIG. 4).

Remove the shift lever detent plate #19.

Remove the sector #26 through the lock cylinder hole by pushing firmly on the block tooth of the sector with a blunt punch (See FIG. 5).

Reassembly instructions follow directly.

# The following procedures address replacing parts and reassembling the steering column.

# Reassembly – Sector, Rack, & Bolt

Insert the new sector through the housing lock cylinder hole and assemble it onto the pin.  Orient the sector so that the tang end is toward the outside of the hole.  Press the sector over the pin with a blunt tool.  The sector should turn freely when installed.

Insert the rack preload spring (See FIG. 6).  The bow of the spring should be against the rack.  Note:  the spring is symmetrical so there is no up or down.  Assemble the bolt to the rack cross-arm.

Insert the rack and lock bolt into the housing.  Assemble the rack to the sector so that the teeth match.  Make sure that the large tooth on the sector matches the large tooth cutout on the rack.

# Reassembly – Housing and Shroud

On floorshift models, position the shroud into the housing.  Drive three screws tighten to 18 inch-lbs.

Place the ignition switch actuator rod into the rack. (Short end of the rod into the rack).

Position the housing (and shroud assembly) onto column jacket and drive screws.  Tighten to 50 inch-lbs.

Continue with Reassembly Instructions Contained in Papers #2 and #1.

Final Words of Caution:

To maintain the energy absorbing function of the steering column, always replace screws, bolts, and nuts as specified.

When a steering column assembly is removed from the car, special care must be taken in handling it.  A sharp blow on the end of the steering shaft, leaning on the assembly, or dropping the assembly could shear or loosen the plastic fasteners that maintain column rigidity.

Plastic parts that are 30 years old can be very brittle!  Handle your steering column parts with care.

StdColumnD&R#315JA03

JIML82@aol.com