Last Revised:  22SE2005

GM Power Steering Hose End Connections (as I remember them for GM vehicles):

C2/C3 Corvette Specific Hoses with Control Valve on the Linkage

1963 through 1979

# Pump to Control Valve - High Pressure Hose

Pump End - 3/8 inch dia tubing with a 5/8-18 UNF male nut, 45 degree flare to mate to the female port in the pump discharge fitting.

Control Valve End - 1/4 inch dia tubing with a 7/16-24 NS male nut, 45 degree flare to mate to the female port in the control valve.

Control Valve to Power Cylinder Hoses (2)

1/4 inch dia tubing with 7/16-24 NS male nuts, 45 degree flare on both ends of the two hoses.

# Control Valve to Reservoir - Low Pressure Return Line Hose

Control Valve End - 3/8 inch dia tubing with a 5/8-18 UNF male nut, 45 degree flare to mate to the female port in the control valve.

# C3 Corvette 1980 through 1982

# Pump to Control Valve - High Pressure Hose

3/8 inch dia tubing with a 16x1.5 metric male nut, o-ring connection to mate to the female port in the pump discharge fitting.

All other connections remained the same as above.  They did not change to metric.

# All Other GM Gear and Pump Ports

1).  Some Chevrolet model power steering pumps from 1963 through 1969 had a male discharge fitting on the power steering pump.  The high pressure hose required a female nut and 45 degree flare.  I don't remember the size of the thread.

2).  All GM integral power steering gears before 1980 had female

11/16-18 NS 45 degree flare high pressure inlet ports.  They had female 5/8-18 UNF     45 degree flare low pressure outlet ports.

3).  All GM integral power steering gears and rack and pinion steering gears from 1980 to present had female M18x1.5 o-ring high pressure inlet ports.  They had female M16x1.5 o-ring low pressure outlet ports.

# Hoses for Hydroboost Brake Units produced before 1980 for GM

Power Steering Pump to Hydroboost Inlet - High Pressure Hose

Pump End – dependent on the pump discharge fitting.

Booster End - 3/8 dia tubing with a 11/16-18 UNS male nut, 45 degree flare to mate to the female port in the booster unit.

Hydroboost Outlet to Power Steering Gear or Control Valve - High Pressure Hose

Booster End - 3/8 inch dia tubing with a 5/8-18 UNF male nut, 45 degree flare to mate to the female port in the booster.

Gear or Control Valve End – dependent upon the high pressure inlet port configuration.

Hydroboost Outlet to Pump Reservoir - Low Pressure Hose

I believe that it was just a 3/8 dia spout to mate to the 3/8 ID return line hose.

# Hydroboost Brake Units from 1980 to present for GM

Power Steering Pump to Hydroboost Inlet - High Pressure Hose

Pump End – dependent on the pump discharge fitting.

Booster End - 3/8 dia tubing with a M18x1.5 male nut, o-ring connection to mate to the female port in the booster unit.

Hydroboost Outlet to Power Steering Gear or Control Valve - High Pressure Hose

Booster End - 3/8 inch dia tubing with a M16x1.5 male nut, o-ring connection to mate to the female port in the booster.

Gear or Control Valve End – dependent upon the high pressure inlet port configuration.

Hydroboost Outlet to Pump Reservoir - Low Pressure Hose

I believe that it was 3/8 dia tubing with a spout to mate to the 3/8 ID return line hose.

JIML82@aol.com

HydraulicPowerStrgConnections.doc