Revised:  07MR2006

# STEERING WHEEL & STEERING SYSTEM CENTERING PROCEDURE (REAR STEER VEHICLES – C2/C3 CORVETTES & GEN I CAMAROS 67-69)

## Procedures Addressed in this Paper					Page

SETTING THE GEAR EXACTLY ON CENTER				  2

CHECKING CONDITION OF FRONT END COMPONENTS		2 & 3

STRG COLUMN SHAFT TO STRG WHEEL HUB ALIGNMENT	3 & 4

DRIVE EVALUATION							4 & 5

ADJUSTING TIE ROD LENGTH						   5

This procedure is valid for C2/C3 Corvettes & Gen I Camaros (1967-1969).  Also other GM vehicles with steering systems where the steering linkage tie rods connect to steering arms that are located behind the centerline of the road wheels (commonly known as rear steer vehicles).

All of the steering parts (i.e. steering linkage, pitman arm, steering gear, flexible coupling, steering column, steering hub, and steering wheel) were designed by General Motors and Saginaw Steering Gear Division such that they could be assembled at high volume and with great integrity, reliability, and dependability.  They were designed so that they only went together one way or they were marked so that the people putting them together had visual indication as to how to correctly assemble them.

It is very important that steering components be in good condition and assembled correctly for the following reasons:

First of all, the steering gear is designed so that when the car is driven straight down the road, the steering gear should be exactly on center.  This is because the gear teeth inside the steering gear were designed with slight interference or a “high spot” exactly on center to insure a crisp and precise steering feel.  About one half turn either side of center, the gear mesh actually has a small amount of clearance designed into it.  If your steering gear is not exactly on center when the car is driving straight, you could be feeling some “looseness” in your steering from this designed-in gear mesh clearance.

Second, the steering column is designed such that the turn signal switch will cancel equally from right and left turns with the column right on its design center position.

Third, the left and right tie rods depend upon the gear being on center so that they will be of roughly equal length with the proper front wheel toe settings.

Because most of us were not privileged to be around for the approximate 30 year history of our cars, we really don’t know if Bubba messed with any of the steering components.  For this reason I have provided some Visual Identification (V-ID) clues to help assure that components are correct.

Setting the Steering Gear Exactly On Center

Before beginning any inspection or adjustment procedure it is best to have the steering gear right on center.  Here is the procedure.  Turn your steering wheel from full lock to full lock.  Carefully count the number of turns and partial turn (X).  Now take that number and divide it in half (X/2).  Starting at full lock, come back exactly X/2 turns.  Your steering gear is now exactly on center.  Take a piece of chalk or a crayon and mark the flexible coupling right at its top most position (12 o’clock.)  With the marks you can just peak under the hood and know when your gear is right on center.  Please note that clock positions are given as you look down the steering column from the driver’s position.

Picture taken looking straight down at the steering gear, flexible coupling, and steering column flange.  Components shown with the steering gear exactly on center.

Checking the Condition of Front End Components

You will need to have a floor jack where you can easily raise and lower your car so that you can get under it for inspections and to adjust the tie rod lengths.  PLEASE USE JACKSTANDS TO SUPPORT THE CAR AS WELL.  It is also a good idea to have the car in a location where it can be let down and easily driven forward a short distance when adjusting tie rod lengths.  This paper assumes that the front end is aligned acceptably.

Checking the Condition of Front End Components (Continued)

First you should inspect your front end and all steering components to look for obvious problem areas.  Look for such things as a bent frame, damaged tie rods, control arms, or relay rod, bent steering arms, pitman arm, etc.  One possible hidden condition can be the steering gear sector shaft being twisted from a severe front impact.

(V-ID) With the gear exactly on center, the pitman arm should be pointing straight  forward.

(V-ID) With the gear exactly on center, the bolt that attaches the flexible coupling to the gear should be passing through the 9 o’clock position on the gear input shaft with its head sticking straight up as shown in the photo on page 2.  The stop pins on the flexible coupling should be vertical (one at 6 o'clock and the other at 12 o'clock).  Note, in the photo, the stop pins have red plastic alignment spacers on them.  Once the steering column is aligned and assembled into the car, the spacers should be pulled off the pins and thrown away

(V-ID) Up until early 1969 the steering gear input shaft and the flexible coupling had what is called “full serrations.”  This allowed the flexible coupling to be installed on any of the serrations on the gear input shaft.

Early in the 1969 model year, steering gear input shafts and flexible coupling flanges were changed so that they had mating flats.  The flat on the gear input shaft will be at 12 o’clock when the gear is exactly on center.  You can see a portion of the input shaft flat sticking out from the flexible coupling flange in the photo on the previous page.

The flats insured that they could only assemble together one way.  So it is possible to have the flexible coupling incorrectly indexed on the gear on vehicles built before 1969.  Also, it is possible to mix and match an early flexible coupling (one with full serrations) such that it will install on any of the serrations of a later gear with a flat.

With the gear exactly on center, now is a good time to check the position of your pitman arm.  Go under the car and look.  It should be pointing straight forward along the centerline of your car.  As noted above, it is possible to actually have a twisted sector shaft from an accident in the past.  If you pull the pitman arm, it is sometimes possible to see the twist in the sector shaft splines.  Sometimes the pitman shaft can be twisted inside the gear box.

# Steering Column Shaft to Steering Wheel Hub Alignment

First check to determine that your steering wheel hub is assembled to the steering column correctly.  There should be a chisel mark on the very end of the upper steering shaft and a corresponding mark on the steering wheel hub.  On standard (non-adjustable) or tilt columns remove the horn cap (or pad) and remove the steering wheel nut and look at the end of the upper steering column shaft.  On vehicles with T&T columns, there are a few more parts to remove.  You must first remove the horn cap, horn contact, shim(s), “star” screw, and telescope lock.

# Steering Column Shaft to Steering Wheel Hub Alignment (Continued)

(V-ID) With the steering gear exactly on center, the chisel mark on the end of the steering column upper shaft should be at the 12 o’clock position and the steering column will be right on its design center position.  If the chisel mark is NOT at 12 o’clock the turn signal switch will not cancel equally left turn versus right turn.

(V-ID) The marks on the steering column upper shaft and the steering wheel hub should be right in line with each other.  If they are not, there is a problem somewhere in the steering or suspension system.  If they aren't aligned, you should pull the hub and align the marks before beginning any centering procedures.

(V-ID) On T&T or tilt columns, with the chisel mark at 12 o’clock the lower end of the column must have its clamp bolt horizontal and passing through the 12 o’clock position on the column shaft (As shown in the photograph on page 2.)  Note that there is a notch on the steering column lower shaft that allows the clamp bolt to pass through.  So the chisel mark on the upper steering column shaft and the notch on the lower steering column shaft must both be at 12 o’clock when the steering column is on center.

If the clamp bolt is horizontal and on the bottom of the shaft (the 6 o’clock position) when the chisel mark on the upper shaft is at 12 o’clock, Bubba has incorrectly assembled the upper to lower steering shafts inside the steering column by 180 degrees.  You have a choice of either of two repair procedures:

1).  Remove the steering column from the car and disassemble the steering column head so that the upper steering shaft can be reoriented to the lower column shaft at the plastic tilt sphere.  It will be very helpful to download the appropriate adjustable steering column disassembly and repair papers from the host websight for instructions.

2).  Disconnect the two nuts and lock washers that attach the flexible coupling to the steering column flange.  Pull the steering column back into the car a short distance.  Remove the flexible coupling from the gear.  There are two different size bolts that attach the flexible coupling to the steering column flange.  With the gear on center, the larger bolt (3/8-24) should be at the 9 o’clock position.  The smaller bolt (5/16-24) at the 3 o’clock position.  Interchange them.  This will rotate the steering column shaft 180 degrees and the turn signals will cancel correctly.

# Drive Evaluation and Tie Rod Adjustment

Now drive your car a short distance on a flat surface in order to determine the steering wheel and gear position when it is traveling a straight path.  With the front wheels in the straight ahead position, now check the mark on the flexible coupling.  The mark should be right at the top (at the 12 o’clock position.)  If it is not on center, then the gear has somehow been moved off its center position and you will now need to adjust the tie rods to reposition the gear back so that the mark is again at 12 o’clock.

Drive Evaluation and Tie Rod Adjustment (Continued)

If your steering wheel needs to be rotated clockwise to bring the gear on center and the mark on the flexible coupling to the 12 o’clock position, you will need to shorten the left tie rod assembly and lengthen the right tie rod assembly.  If your steering wheel needs to be rotated counterclockwise to bring the gear on center, the left tie rod assembly will need to be lengthened and the right one shortened.

Loosen the tie rod adjuster tube clamps on both the left and right tie rods, then turn both tubes an equal number of turns in the same direction to bring the gear back on center.  DO NOT turn the sleeves an unequal number of turns.  If you closely follow this procedure you will not change your front toe setting.  If you turn the tubes in one direction and the steering wheel position gets worse, start turning them in the opposite direction.  Just make sure that you rotate them the same direction and the exact same number of turns.

For a more complete explanation as to C3 Corvette tie rod ends, clamps, and adjuster tubes, you can refer to the following paper and pictures entitled Steering Linkage Tie Rod Adjustment Specs Explained - Revised 21JA06Link .  This paper explains the engineering terminology on the C3 AIM sheet which displays how to correctly orient and tighten the tie rod clamps at the end of the adjustment procedure.

I have not been able to find any information on the correct orientation of the tie rod clamps for Gen I Camaros.  Please refer to your AIM sheets or other shop manuals for this information.

Final Steering Wheel and Tie Rod Adjustments

With the wheels straight ahead and the gear on center, check the steering wheel alignment.  The 6 o’clock steering wheel spoke measured at the steering wheel rim should be within 1 inch of being exactly at the bottom.  If the wheel is not in alignment, remove it and try to reindex the hub on the steering column shaft by one spline to center the wheel as closely as possible. With 30 serrations on the steering wheel hub and steering column shaft, you should be able to get the steering wheel centered within 6 degrees or less.

Now, if you want to fine tune the steering wheel to get it perfectly on center, you will have to adjust the tie rods one last time.  If the steering wheel needs to be adjusted clockwise to get it on center - the left tie rod will need to be shortened and the right lengthened by equal amounts.  If the steering wheel needs to be adjusted counterclockwise – the left needs to be lengthened and the right shortened.

SteeringSystemCenteringC3-Rev 07MR2006

JIML82@aol.com