Last Revision: 28JN2007

1969 THROUGH 1982 CORVETTE TILT & TELESCOPING STEERING COLUMN DISASSEMBLY & REPAIR INSTRUCTIONS

#### PAPER #3

Disassembly and Repair Instructions Addressed in this Paper

Degree of Difficulty   Page

REPLACE THE WASH/WIPE SWITCH (1977 ONLY)		Difficult	    2

### REPLACE THE DIMMER AND TURN SIGNAL LEVER		Difficult	    2

PIVOT (1978 THROUGH 1982)

REPAIR & REMOVE LOOSE PIVOT PINS			Hardest	  3 & 4

REPAIR LOOSE LOCK SHOES					Hardest	    4

REPAIR RACK AND SECTOR					Hardest	    5 REPAIR THE UPPER BEARINGS					Hardest	    5

TIGHTEN SUPPORT SCREWS					Hardest	    6

ELIMINATE LASH IN THE PLASTIC TILT JOINT		Hardest	 6 & 7

# STEERING SHAFT LENGTH CHECK				Hardest	    7

# How the Paper is Setup

This is the third of a series of three papers that address various replacement and adjustment procedures that can be performed on the Corvette C3 tilt and telescoping (T&T) steering column.

This paper starts at the point where the steering column has been removed from the car.  Also, the steering wheel, horn parts, shaft lock, ignition lock cylinder, key warning buzzer, turn signal, ignition, and dimmer switches have been removed from the T&T steering column.  All of these operations were described in detail in Papers #1 & #2.

This paper makes reference to Descriptions #4, #5, #6 & #7.  They are shown in various sections on a three pages of line drawings entitled Corvette C3 Tilt & Telescoping  Steering Column Page - #1, #2, & #3.

Also there is a schematic drawing entitled Corvette C3 T&T Steering Column Blowup.  Most steering column parts will be called out with a reference number from this blowup.

Please note that the blowup drawing shows a 1969-76 column and does not show the dimmer switch, the special housing with the dimmer pivot, or the finger key release mechanism used on the 1977 through 82 C3s.

The drawings are all available from the author or from the host websight. You will find these pictures and descriptions to be most helpful when working on your steering column.

The T&T steering columns from 1969 through 1976 are called “round” columns because the column head is completely circular in shape.  Starting in 1977, a characteristic bulge appeared on the left side of the column head.  This bulge housed the headlight dimmer pivot which actuated a rod which tripped the dimmer switch now mounted on the steering column jacket down under the dash.

# Disassemble the Column Head - Description #4

Unscrew and remove the tilt lever.  Remove the three turn signal housing screws #12 located at the 12, 3, and 8 o’clock positions .

On 1977 and later columns the cover #15 will also contain a dimmer pivot assembly, a wash/wipe switch (1977 only) and a dimmer rod actuator which will come with the cover and its end cap.  Remove the cover from the housing.  Be careful of the end cap and dimmer switch rod actuator.  Separate the actuator from the cap by simply pulling them apart.  They are held together by grease.

Guide the (1977) wash/wipe wires out of the column as you remove the cover.  It is a good idea to tie tracer string to the wiring connector to aid when you reassemble it.

# Replace the Wash/Wipe Switch (1977 only) and/or the Dimmer Pivot (1978 thru 82)

The wash/wipe switch/dimmer pivot is held to the inside of the cover by a pivot pin. You must feed the wash/wipe switch wires back through the cover (1977 only).  Then use a punch to remove the pin.  The switch/pivot can be pulled from the cover.  The 1977 wash/wipe switch/dimmer pivot #7827252 is no longer available through GM dealers.

# Remove Tilt Spring - Description #5

Replace the tilt lever and place the column in the full “up” position.

Caution: The spring is quite powerful, without the steering wheel to counterbalance its force, the column head can spring upward with a lot of energy.

Remove the tilt lever spring retainer #16 using a tool that just fits into the slot opening.  Note, the picture on page #6 shows a screwdriver blade being inserted into the retainer slot for removal/installation.  Early retainers have a slot, later retainers have a square hole.  A medium size Phillips bit will fit the square shape quite well.

Caution:  The spring is compressed with a lot of force.  Press squarely on the retainer and press in approximately 3/16 inch, turn 1/8 turn counterclockwise until the ears align with the grooves in the housing and remove the retainer, spring #17, and guide #18.

# Remove Dimmer and/or Ignition Switch – Description #7

The dimmer switch and/or the ignition switch should already have been removed from the steering column jacket.  Reference Paper #2; pages 4, 5, & 6.

# Remove Pivot Pin - Description #5

Remove the two pivot pins #35.  Each of the pins has a tapped hole so that you can use the GM tool or jury rig a puller using an 8-32 screw, nut, washer, and a ½ inch or a 12mm socket.  Bridge each pin with the socket and use the screw and nut to pull them out.  You will note that the bowl of the transmission control lock tube housing #58 comes right up to the edge of each pivot pin.  You cannot react against the fragile bowl.  But you also must keep your removal tool square to the pivot pins.  Two home made tools that provide clearance to the bowl and keep things square are shown on the next page.

# One other method to extract the pivot pins would be to thread a 8-32 screw into the end of each pin.  Grip the screw with a slam puller tool.  A few quick raps should draw the pin right out.

# Remove Bearing Housing – Description #5

Remove the housing by pulling upward on the tilt lever and pull the housing upward until it stops.  Move the housing to the right or left to disengage the rack #37 from the ignition switch rod #38.  Pull the housing up off the steering shaft assy.

Loose Pivot Pins

The pivot pin large diameter (0.4000/0.4005 inch) should be a press fit to the housing #24.  The pin smaller diameter (0.3745/0.3740 inch) should be a sliding fit to the support #49.  The pivot pin holes in the support and in the housing as well as the diameters of the pivot pins are held to very precise tolerances.

The large diameter on the pin should be an interference fit to the housing.  It should be impossible for you to insert it into the housing by hand.  You will have to tap it with a hammer or press it in place with a c-clamp or other devise.  Once a pin is pressed in place in the housing, it should be secure with absolutely no looseness.

You should be able insert the smaller diameter of the pivot pin into the support hole.  Now, thread an 8-32 screw all the way into the tapped hole in the pivot pin.  Secure it in place with a jam nut.  Now, try and wiggle the screw, you should not feel any motion between the pivot pin and the support.  If one or both holes in the support are badly out of round or tapered, it is probably best to replace the support.

If there is any detectable play at the pivot pin to support or at the pivot pin to housing interface, you will feel play in the steering column head in the side to side direction.

Loose Pivot Pins – Possible Corrective Actions

1).  The pivot pins are hardened.  They also have a very thin surface coating.  The first thing to try is to rotate the loose pivot pin(s).  It just might be possible that the surface coating on the pin may be worn in just one location so that rotating the pin to a different position and reinserting into the housing or support might regain the precision fit.

2).  The next approach may be to replace the housing or support with one that has pivot holes of the correct size.  The support #49 from any tilt or T&T column from 1969 through at least 1982 should be an acceptable replacement.  However, only a T&T column with a round head can supply the housing #24 for a 1969 through 76 T&T column.  Only a T&T dimmer column (one with the bulge on the left side of the housing) can supply a replacement housing for a 1977 through 1982 column.  T&T columns were also available in Cadillacs and big Oldsmobiles from 1969 through the 1970s.

3).  The other course of action is to make the pivot pins larger.  If you have access to a machine shop, you could fabricate new pivot pins from hardenable steel with a larger outside diameter to regain the press fit to the housing and/or the precision slip fit to the support.  Another thing you could try is to plate the original pivot pins.  After plating you will probably have to polish the appropriate diameters so as to attain the proper interference or slip fit.  Remember, a press fit is only a few ten thousandths of an inch of interference between the pins and the housing holes.  Any more interference and the pins will plow metal out of the mating housing holes as they are being inserted rather than just expanding the hole to lock them in place.

Loose Lock Shoes

There are two lock shoes #25 in every tilt or T&T steering column.  They alternate locking the steering column tilt head one at a time.  The Corvette T&T column has one shoe with four engagement slots while the other shoe has three slots.  This results in seven tilt positions in a Saginaw adjustable steering column.  That is why if you have one bad shoe or the pin is worn where only one shoe pivots, it is possible that your column head may feel loose at every other tilt position.

Both shoes pivot on a small pin #27 that is part of the housing.  They engage a pin #50 that is part of the support.

Look for a worn shoe pivot pin.  The shoes should rotate freely on the pin but there should be no noticeable looseness.  Replace the pin and/or the shoes if needed.  DO NOT LUBRICATE THE SLOTS IN THE LOCK SHOES.  They should be “dry as a bone”.  It is possible that the column can be made to “ratchet” all the way to the lowest tilt position if the slots are lubricated.

If you have access to a donor tilt or T&T steering column, you may find that the lock shoes do not have locking slots in exactly the same location as the shoes from your Corvette T&T.  The Corvette column had the maximum amount of tilt adjustment travel of any of the GM columns.  Tilt shoes from other adjustable columns will still fit your housing but may slightly restrict your maximum up or down tilt travel.

# Remove Rack and Sector – Description #5

Remove the tilt lever opening shield #20 from the housing and also the turn signal lever opening shield if so equipped.  Remove the lock bolt spring #23 by removing the spring retaining screw #32, and moving the spring clockwise to remove the lock bolt #22.

If there is a snap ring #33, remove it from the sector drive shaft #26.  With a small punch lightly tap the drive shaft from the sector.  Remove the drive shaft.  Remove the rack and rack spring (also a shim, if there is one).  Remove sector and bolt.

Replace the Upper Bearings – Missing Balls/Broken Separator - Description #5

There are two bearings (#21 & #39) located in the bearing housing #24, they are seated back to back with one inch between them.  They consist of an outer race, a plastic ball separator, and ball bearings.  The bearings are press fit into the housing, one toward the driver and the other one in the other side of the housing pointed in the opposite direction.  The balls ride on inner races.  One is permanently pressed onto the steering shaft yoke #44.  The other inner race #10, the one closest to the driver, is a loose fit to the shaft.  This one is preloaded by upper bearing inner race seat #9 that is pushed by a 100# spring #5, and secured with that D-shaped shaft lock retainer clip #2.

The column upper bearing service kit is GM part number 26001827.  It contains one bearing with an inner and outer race.

However, if you need both bearings, I have found that it is cheaper if you purchase GM kit #7844651.  The kit contains two bearings, along with a rack, sector, lock bolt spring, and screw.  Just save the extra parts or replace your thirty year old rack, sector, and spring while you have the housing apart.  You should be able to purchase either kit from any GM dealer.

One other suggestion.  One enthusiast found very similar bearings at a bicycle shop!  I would guess that they were probably a lot cheaper than going through a dealership.

Now that you have the housing removed from the column, place it on a terry cloth surface.  You now need to take a small screwdriver and pop the individual ball bearings out of the separator.  You will soon understand why I recommended a terry cloth work surface.  With all of the balls removed, it is now possible to inspect the plastic separator.

If it is brittle and broken, replace it by reaching into the housing and popping the separator out of the inner race.  If it is in good condition, leave it alone.

Inspect the inner race, if it is in good condition, leave it alone.  Otherwise, you need to reach through the back side of the housing with a thin punch and using the notches that are in the housing bore to assist you, work the race out of the housing.  Press the new

inner race into place.  Caution:  Do not press the race into the housing by pushing on the balls, this will cause them to damage the race and rotation of the steering shaft will feel rough.  Snap the new separator into the inner race.  Individually snap the ball bearings into the separator.  Make sure that you lubricate the bearing with the grease that comes in the service kit.

Tighten Support Screws - Description #7

If you have come this far, be sure to retighten the support screws.  If your column head hasn’t loosened as yet, it still probably will in the future.  (This has been a common problem.)  And who wants to go through this again!

You should now locktite and tighten (88 in-lbs, 7 ft-lbs max) the four screws #48 that hold the support #49 to the support plate lock #55 and to steering column jacket #63.  You don’t have to disassembly everything, apply the locktite one bolt at a time.  Then torque them sequentially in 20 in-lb increments.

# Reassembly – Sector, Rack, Leaf Spring, & Bolt

Install the drive shaft into the housing.  Lightly tap the sector onto the shaft far enough to expose the snap ring groove.  Replace the snap ring if it was removed.  On later columns, lightly tap the sector onto shaft far enough to snap sector into drive shaft groove.

Install the lock bolt and engage with the sector.  Install the rack and leaf spring.  (Replace shim if one was removed.)  The big tooth on the sector must engage the corresponding big tooth on the rack.

Reassembly – Lock Bolt Spring

Install the lock bolt spring and spring retaining screw.  Tighten screw to 35 inch lbs.  Install the ignition switch actuator rod through the shroud from the top and insert it in the slot in the support.

Steering Shaft Tilt Joint Lash Check – Description #6

If you have removed the steering shaft assembly from the column, here is a quick test to determine if the plastic ball tilt joint is properly snug.  Straighten the shaft assembly at the pivot ball and hold the lower steering shaft as the upper steering shaft sticks straight out.  There should be sufficient friction in the plastic ball to the shaft yokes so that the upper shaft will remain straight or will just barely slump downward.  If the joint is so loose that the upper shaft falls to a 90 degree angle, most likely you will have some looseness in the joint where you might detect some steering wheel play while driving.

Steering Shaft Tilt Joint Lash Check (Continued)

To regain the proper amount of tightness you will need to separate the upper from the lower steering shaft.  First mark the upper and lower shafts so that you can reassemble them with the correct upper to lower shaft orientation.  Now take a mallet and tap on the lower shaft yoke ears.  Do not tap on them too hard!  Reassemble and try the “slump test” again.  Continue tapping on the yoke ears until the looseness in the ball joint is eliminated.

Steering Shaft Length Check

It is a good idea to check the length of the lower steering shaft #47.  There are two dimensions that can be checked; one while it is out of the column, the other when it is assembled into the column.  It is possible that the shaft could be compressed during handling or when the steering column was removed from the car.  The lengths for the complete shaft are from the center of the yoke to the end of the shaft.  If there is a question as to exactly where the yoke center might be located, place one half of the plastic centering sphere #46 in the yoke and measure from the flat surface.

Steering Shaft Length

Complete Shaft			Shaft Extending from Column

1969 through 1976   31.0 inches	1968  2.21 inches to edge of clamp

1977 through 1982   30.1 inches	1969 through 1976  4.70 inches to brg retainer face

1977 and 1978         4.90 inches to brg retainer face

1979 through 1982  5.00 inches to brg retainer face

Steering Shaft Orientation - Reassembly Tip

Notice:  If for some reason you lose the original orientation of the steering shaft yoke #44 to the lower shaft #47, you will find that you will have two possible ways to reassemble this connection.  If you look closely at the steering shaft yoke, you will note that the end that points at the steering wheel has two machined flats, one on each side.  However, one flat is longer than the other.  This is important because the carrier #4 has a flat that must mate with the long flat.  The carrier also has the turn signal cancelling tabs molded into it.

Steering Shaft Orientation - Reassembly Tip (Continued)

If you reassemble the shaft yoke to the plastic spheres in the wrong orientation, your turn signals will not cancel properly.  (You probably won’t notice this problem until you have the column back in the car!)

Here is the key to the correct alignment.  Look down the column from the steering wheel end, place the pinch bolt notch (located on the very lower end of the column, where the steering column flange is connected) at the 12 o’clock position.  Now when the upper steering shaft is connected to the plastic tilt centering spheres, the long flat should be at the 6 o’clock position.

Reassembly - Housing to Support & Pivot Pins

Screw the tilt lever into the shoe release lever #31.  While holding up the tilt lever to disengage the lock shoes, “walk” the housing onto the support.  Be very careful when you slide the housing over the steering shaft so as not to displace any bearing balls in the housing.  Move the rack downward and hold.  Tip the housing to the left until the rack engages the pin on the actuator rod.  If this is a dimmer column, snake the dimmer rod through the opening in the housing at the 9:00 position.

Push the housing down until the pivot pin holes are in alignment.  Release the tilt lever so that one of the lock shoes will engage the dowel pin.

Lubricate and install the pivot pins.  Use palm pressure to start them.  Make sure that the left pin is straddled by the rack.  Use a small hammer and punch to set the pins so that they are flush with the bearing support.  Verify that the column will tilt properly.  Unscrew the tilt lever.

Reassembly – Tilt Spring

Place the housing in the full “up” position.  Assemble the tilt spring #17 and spring guide #18.  Apply grease between the guide and the peg on the support.  Turn the tilt spring retainer #39 clockwise to secure it (shown above.)

Reassembly – Shield

Install the tilt lever opening shield in the housing.

Reassembly - Dimmer Actuator, and Lock Housing Cover

Install the dimmer pivot (and wash/wipe switch – 1977 models) to the lock housing cover.  Route wash/wipe switch wires down through the column.  Assemble the plastic dimmer rod actuator to the end cap using some heavy grease.  The actuator is guided between the end cap and the metal housing and is retained during assembly by the grease on the little square post inside the cap.  Without the grease, the actuator is very difficult to hold in place while reassembling the column because the wire rod isn’t exactly tightly fixed in position.

Assemble the cover with three screws.  Seat the screw at the 12 o’clock position first.  Tighten the three screws to 100 in-lbs.  Screw in the tilt lever.  Install the housing cover end cap.  Verify that the dimmer switch functions (the rod can become disengaged from the switch) before continuing the rest of the column reassembly.

Continue with Reassembly Instructions Contained in Papers #2 and #1.

T&TColumnD&R#3-28JN07.doc

JIML82@aol.com